# EXPERIMENT – 7

CALCULATION	AND	VERIFICATION	OF	FREQUENCY	DEVIATION	AND MODULATION INDEX OF FM SIGNALS USING MATLAB SIMULATION

## PROGRAM:

% Calculation and Verification of Frequency Deviation

% and Modulation Index of FM Signal clc;

clear; close all;

%% Message Signal Parameters Am = 2; % Message amplitude (V)

fm = 500; % Message frequency (Hz)

%% Carrier Signal Parameters Ac = 1; % Carrier amplitude (V)

fc = 10000; % Carrier frequency (Hz)

%% FM Parameters

kf = 250; % Frequency sensitivity (Hz/V) fs = 100000; % Sampling frequency (Hz) T = 0.01; % Simulation duration (s)

%% Time Vector t = 0:1/fs:T;

%% Message Signal

m = Am*cos(2*pi*fm*t);

%% Frequency Deviation delta_f = kf*Am;

%% Modulation Index beta = delta_f/fm;

%% FM Signal

fm_signal = Ac*cos(2*pi*fc*t + beta*sin(2*pi*fm*t));

%% Display Parameters

fprintf('Carrier Frequency (fc) = %.0f Hz\n',fc); fprintf('Message Frequency (fm) = %.0f Hz\n',fm); fprintf('Message Amplitude (Am) = %.0f V\n',Am); fprintf('Frequency Sensitivity (kf) = %.0f Hz/V\n',kf); fprintf('\nFrequency Deviation (Delta f) = %.2f Hz\n',delta_f); fprintf('Modulation Index (Beta) = %.2f\n',beta);

%% Verification beta_verify = delta_f/fm;

fprintf('Verified Modulation Index = %.2f\n',beta_verify);

%% Verification Check tolerance = 1e-6;

if abs(beta - beta_verify) < tolerance fprintf('\nVerification Successful\n'); else

fprintf('\nVerification Failed\n'); end

%% Plotting figure; subplot(2,1,1);

plot(t,m,'LineWidth',1.5); grid on;

title('Message Signal'); xlabel('Time (s)');

ylabel('Amplitude (V)');

subplot(2,1,2); plot(t,fm_signal,'LineWidth',1); grid on;

title('FM Signal');

xlabel('Time (s)');

ylabel('Amplitude (V)');

## OUTPUT: